Character received?

Start

No

Process Received Character

Routine called in a timer event

1

See:

State 0

State 1

State 2

State 3

State 4

Char != ‘5’ ?

Char = ‘5’ ?

- send ‘A’

- update status messages

Char != ‘5’ ?

Char = ‘5’ ?

- send ‘U’ ‘3’

- update status messages

Char != ‘R’ ?

Char = ‘R’ ?

- send Start Address, Code Size, and actual Code to be downloaded into iRAM.

- update status messages

State 5

(finish)

Char = ‘e’ OR ‘E’ ?

- Release COM port

- update status messages (inform error)

Char != ‘D’ AND Char != ‘e’ AND Char != ‘E’ ?

Char = ‘D’ ?

- send NOR Flash Start Address, and user’s Code Size of the image to be burned into NOR Flash.

- if’ Validation Word’ option is checked, increment code size by 4

- update status messages

Char = ‘E’ ?

- Release COM port

- update status messages (inform error)

Char = ‘F’ ?

- Release COM port

- update status messages (inform success)

Char = ‘S’ ?

- If ‘Val_Word’ checked and NOT yet sent, send it.

- Send NEXT 32K of user’s code

1

Process description

Notes:

- Each time Start button is pressed, the State variable is reset to 0.

- The conditions that trigger a state change are in Bold, while the actions taken in the state transitions are in Italic.

- During States 0, 1 and 2 (light yellow), the handshaking occurs between UART5 Boot ROM process and the Utility. During States 3 and 4 (light green) the handshaking occurs between Loader code (Loader Keil’s project) running on iRAM and the Utility.